# Common Set Proposal

# Proposed Scene Pool

- CSRC 01, “NTIA outdoor mall with tulips (5e)” from www.cdvl.org  1080p 25fps 14sec

- CSRC 02, “NTIA Power Dig” from www.cdvl.org  1080p 30fps 14 sec

- CSRC 03, “NTIA pedestrians cross street on a snowy day (4a)” 1080i 59.94fps last 14 sec

- CSRC 04, SVT 1080p 50fps 14sec, frames [3501 to 4450].  Download:

  - “SVT 1080p 50fps frames 3001 to 4000 of 18677” from www.cdvl.org

  - “SVT 1080p 50fps frames 4001 to 5000 of 18677” from www.cdvl.org

  - Take frames [3501 to 4200]

  - Convert to 1080i 50fps for original

# WVGA/VGA PVS Creation

- Convert CSRC 01, 02, 03, and 04 to WVGA.  This will be the original format.

  - Deadline: January 12

- PVS creation:

  - FUB creates 5 PVSs from CSRC 01 (wvga), that span the quality range seen in WVGA2

  - Yonsei creates 5 PVSs from CSRC 02 (wvga), that span the quality range seen in WVGA1

  - Swissqual creates 5 PVSs from CSRC 03 (wvga), that span the quality range seen in VGA1

  - Opticom creates 5 PVSs from CSRC 04 (wvga), that span the quality range seen in VGA3

  - All laboratories should check that the WVGA and VGA versions of each PVS appear to have identical quality

  - Deadline: February 28

- Format conversion:

  - The 4 SRC x (5 PVS + original) will be format converted to VGA resolutions by chopping off the left and right side.

  - The 4 SRC x (5 PVS + original) will be frame-rate converted (25fps to 30fps; 30fps to 25fps).

  - The appropriate format converted versions will be included in each subjective test.

- WVGA/VGA Super set:

  - Within the super-set, each CSRC’s scores will be taken from the specific dataset as follows:

    - WVGA1:  CSRC 02 & 03 (30fps)

    - WVGA2:  CSRC 01 & 04 (25fps)

  - These will be compared with the model only within the context of the WVGA/VGA super-set.

# HD PVS Creation

- Original CSRC format is as shown above.

  - Deadline: January 12

- PVS Creation

  - Ghent creates 5 PVSs from CSRC 01, that span the quality range seen in HD5

  - DT creates 5 PVSs from CSRC 02, that span the quality range seen in HD4

  - RT-RK creates 5 PVSs from CSRC 03, that span the quality range seen in HD1

  - RT-RK creates 5 PVSs from CSRC 04, that span the quality range seen in HD3

  - Deadline: February 28

- Format Conversion

  - The 4 SRC x (5 PVS + original) will be format converted to all target HD resolutions and bit-rates, for inclusion into each subjective test.

- Within the super-set, each CSRC’s scores will be taken from the specific dataset mentioned above.  These will be compared with the model only within the context of the HD super-set.